Thomas Edison's Filament Experiment.

Equipment:

- Nichrome wire		10 feet of both 32 and 28 gauge wire

(Wiretronic inc. 1-888-947-3876)

- Copper wire		20 feet of wire

- Mason Jars		Six jars (found in grocery stores for canning fruit)

- Tea light candles	Six candles (and matches to light the candles)

- Batteries			At least ten 6 volt batteries

- Wire cutters/strippers

- Scotch Tape

Set-up:  (See example below)

- Punch two holes in the metal cap of a mason jar.

- Get two pieces of copper wire about 6 inches in length.

- Use a wire stripper to strip the insulation off both ends of the wires.

- String one of the copper wires halfway through a hole in the jar lid.

- Tape the wire down to the top of the lid with scotch tape.

- Repeat the last two steps with the other copper wire.

- Cut lengths of nichrome wire (about 2 inches) to test.

Directions:

- Unscrew the lid of the mason jar.

- Get a length of nichrome wire and attach it to the two copper wires.

- Screw the lid back on the jar.

- Attach a 6 volt battery to the wires on the outside of the jar and see if the filament will light.

- Try a different gauge of wire.  Does it work better, last longer, Etc.?

- What would happen if you could get rid of the oxygen in the mason jar before lighting up the filament?  How could you do this?  (Hint: candle)

- Try different batteries or more batteries in series or parallel. What happens?  What else could you test?